16-642 Manipulation, Estimation, and Control
Problem Set 2

David Wong (Andrew ID: DBWONG)
collaborator: Kelvin Kang

Q1 (a)

G(s)=200s3+22s2+141s+2

T(s)=200s3+22s2+141s+202

MATLAB code:

K = 1 %negative unity feedback assumed by default for feedback function in MATLAB

G = tf([0 200],[1 22 141 2])

T = feedback(G,K)

Q1(b)

Poles =

-10.0000 + 1.0000i

-10.0000 - 1.0000i

-2.0000 + 0.0000i

No Zeros (function returns a 0×1 empty double column vector)

MATLAB code:
%Q1b find poles and zeros

sys = T

P = pole(sys)

Z = zero(sys)

Q1(C)

Plot y(t) using step. Discuss which poles dominate the response

-2.0000 + 0.0000i

The pole that dominates the response is the one nearest to the imaginary axis. This appears to be a overdamped system.

MATLAB CODE:

%Q1c

step(sys)

Q1(D)

Steady state value with Final Value Theorem

T(s)=200s3+22s2+141s+202

Final value theorem:

lims=0=sTs=200/202

Steady state value is 0.9901

Q2

Rise time = 0.471s
%overshoot = 4%

%Q2

numerator = [1 10];

denominator = [1 71 1070 1000];

sys2 = tf(numerator,denominator)

pidTuner(sys2,'PID')

Q3 Observer for cart-pendulum system

Employed MATLAB to construct observability matrix and used tank command to check for system observability:

Observability = 4. (full rank, hence system is observable)

System poles:

0

-3.3301

1.1284

-0.7984

Therefore observer poles via pole placement(> 5x system poles):

obsPole =   -20   -30   -40   -50

Ko =

1.0e+04 *

0.0004, 0.3956, 0.0172, 3.1123

Expected plot should look something like this (which exhibits a worse tracking performance than the original tracking controller as we are tracking xhat instead of x directly)

Original Tracking controller plot of outputs with desired square wave

Tracking controller state vs time

% MEC PS2

%David Wong (Andrew ID: DBWONG)

%Q1a

clear all;close all;

K = 1 %negative unity feedback; feeback function assumes -1 by default

G = tf([0 200],[1 22 141 2])

T = feedback(G,K)

%Q1b find poles and zeros

sys = T

P = pole(sys)

Z = zero(sys)

%Q1c

step(sys)

%Q2

numerator = [1 10];

denominator = [1 71 1070 1000];

sys2 = tf(numerator,denominator)

pidTuner(sys2,'PID')

%% Q3

clear all;

close all;

%Ref HW1 Q2h

tspan = 0:0.01:200;

gamma = 2;

alpha = 1;

beta = 1;

D = 1;

mu = 3;

R = 10;

Q = [1000 0 0 0 ; 0 5 0 0 ; 0 0 1 0 ; 0 0 0 5];

x_00 = [0; 0; 0; 0];        % initial condition for 2g

A = [0,0,1,0;0,0,0,1;0,1,-3,0;0,2,-3,0]

B = [0;0;1;1]

C = [39,0,0,0];

D = [0,0,0,0; 0,0,0,0; 0,0,0,0; 0,0,0,0];

[K,S,e] = lqr(A,B,Q,R)

[tout,xout] = ode45(@(t,x) trackingController(t,x,K,C,A,B), tspan, x_00);

figure

%plot(tout,xout)

grid

title('Tracking Controller State vs Time')

yout = C.*xout;

xhatdot = A*xhat + B*u - Ko(yout-yhat)

%%PARTIAL CODE

tf=200;

T = 0.01;

yd1= 20*ones(1,(tf/4)/T);

yd2= -20*ones(1,(tf/4)/T);

yd = cat(2, yd1,yd2,yd1,yd2);

figure

plot(yd)

hold on

%plot(yout)

grid

title('Desired vs Actual Output over time')

legend('Desired','Actual')

hold off

% OBSERVER PLACEMENT

sys3 = ss(A,B,C,0)

ob = obsv(sys3)

observability = rank(ob)

sysPoles = eig(A) %system poles

A_transp = transpose(A);

C_transp = transpose(C);

%Kc from LQR :

Kc = [-9.99999999999901, 34.1720722633026, -28.5448907859479, 33.1569959170274]

%define observer poles as >5x dominant system poles

obsPole = [-20, -30, -40, -50]

L = place(A_transp, C_transp, obsPole)

Ko = transpose(L)

At = [A-B*.K, B.*K;

zeros(size(A)), A-L.*C];

Ct = [C, zeros(size(C))];

sys = ss(At,Bt,Ct,0);

lsim(sys,zeros(size(t)),t,[x0 x0]);

%%%%%%%%%%%%%%%%

tf = 200;

step = 0.01;

numIteration = ceil (tf/step);

x = zeros(1,numIteration);

xhat = zeros(1,numIteration);

xdot = zeros(1,numIteration);

xhatdot = zeros(1,numIteration);

zdot = [A, -B*Kc; Ko*C, A-B*Kc-Ko*C]

A_fb = zdot;

disp('Q5.2 resultant Poles of A_fb:')

disp(eig(A_fb));

for k = 1:numIteration-1

k1 = T*(A_fb*x(:,k));

k2 = T*(A_fb*(x(:,k)+0.5*k1));

k3 = T*(A_fb*(x(:,k)+0.5*k2));

k4 = T*(A_fb*(x(:,k)+k3));

x(:,k+1) = x(:,k) + k1/6 + k2/3 + k3/3 + k4/6;

t(k+1) = t(k) + T;

z

end

function xdot = trackingController(t,x,K,C,A,B);

x_2 = x(2);%phi

x_3 = x(3);%xcdot

x_4 = x(4);%phidot

yd=0;

if t<50

yd = 20;

end

if t>=50 && t<100

yd = -20;

end

if t>=100 && t<150

yd = 20;

end

if t>=150 && t<200

yd = -20;

end

ABK= A-B*K;

ABKinv =inv(ABK);

v_partial = -(C*(ABKinv)*B)^(-1);

v = v_partial * yd;

F = v-K*x;

xcddot=(sin(x_2)*x_4^2 - F + 3*x_3)/(cos(x_2)^2 - 2) - (cos(x_2)*sin(x_2))/(cos(x_2)^2 - 2)

phiddot=(cos(x_2)*(sin(x_2)*x_4^2 - F + 3*x_3))/(cos(x_2)^2 - 2) - (2*sin(x_2))/(cos(x_2)^2 - 2)

xdot = [x_3;x_4;xcddot;phiddot];

end